# NWT – Sumer 2026

## LX Patch Plan

Universe 1 – Generics, House Lights, Hot-Power

Universe 2 – Overhead LX1 – LX5

Universe 3 – FOH, Bridge 1, Bridge 2, Cheeky

| Channel No. | Universe | Address | Fixture Type | Notes |
| --- | --- | --- | --- | --- |
| 1 | 3 | 100 | Rouge Outcast 3 Spot [25ch] | FOH HL on Bridge 2 |
| 2 | 3 | 125 | Rouge Outcast 3 Spot [25ch] | FOH HL on Bridge 2 |
| 3 | 3 | 150 | Rouge Outcast 3 Spot [25ch] | FOH HR on Bridge 2 |
| 4 | 3 | 175 | Rouge Outcast 3 Spot [25ch] | FOH HR on Bridge 2 |
| 50 | 1 |  | Source 4 36˚ | Above HL Doorway |
| 51 | 1 |  | Source 4 36˚ | Above HL Doorway |
| 52 | 1 |  | Source 4 36˚ | Above HR Doorway |
| 53 | 1 |  | Source 4 36˚ | Above HR Doorway |
| 54 | 1 |  | Source 4 36˚ | HL Cheeky |
| 55 | 1 |  | Source 4 36˚ | HR Cheeky |
| 56 | 1 |  | Source 4 36˚ | HL Cheeky |
| 57 | 1 |  | Source 4 36˚ | HR Cheeky |
| 60 | 1 |  | Source 4 25˚- 50 ˚ | HL Cheeky |
| 61 | 1 |  | Source 4 25˚- 50 ˚ | HR Cheeky |
| 62 | 1 |  | Source 4 25˚- 50 ˚ | HL Cheeky |
| 63 | 1 |  | Source 4 25˚- 50 ˚ | HR Cheeky |
| 64 | 1 |  | Source 4 25˚- 50 ˚ | HL Cheeky |
| 65 | 1 |  | Source 4 25˚- 50 ˚ | HR Cheeky |
| 101 | 2 | 100 | ColourSource PAR [5ch] | LX1 Overhead ColourSource PAR |
| 102 | 2 | 105 | ColourSource PAR [5ch] | LX1 Overhead ColourSource PAR |
| 103 | 2 | 110 | ColourSource PAR [5ch] | LX1 Overhead ColourSource PAR |
| 104 | 2 | 115 | ColourSource PAR [5ch] | LX1 Overhead ColourSource PAR |
| 105 | 2 | 120 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 106 | 2 | 125 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 107 | 2 | 130 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 108 | 2 | 135 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 109 | 2 | 140 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 110 | 2 | 145 | ColourSource PAR [5ch] | LX2 Overhead ColourSource PAR |
| 111 | 2 | 150 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 112 | 2 | 155 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 113 | 2 | 160 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 114 | 2 | 165 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 115 | 2 | 170 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 116 | 2 | 175 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 117 | 3 | 200 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 118 | 3 | 210 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 119 | 3 | 205 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 120 | 3 | 215 | ColourSource PAR [5ch] | LX3 Overhead ColourSource PAR |
| 201 | 3 | 11 | S4 LED S2 Lustr Direct Fan [10ch] 25˚- 50 ˚ | Bridge 1 USR Front Light |
| 202 | 3 | 22 | S4 LED S2 Lustr Direct Fan [10ch] 25˚- 50 ˚ | Bridge 1 UCS Front Light |
| 203 | 3 | 33 | S4 LED S2 Lustr Direct Fan [10ch] 25˚- 50 ˚ | Bridge 1 USL Front Light |
| 204 | 3 | 44 | S4 LED S2 Lustr Direct Fan [10ch] 15˚- 30 ˚ | Bridge 2 DSR Font Light |
| 205 | 3 | 55 | S4 LED S2 Lustr Direct Fan [10ch] 15˚- 30 ˚ | Bridge 2 DSR Font Light |
| 206 | 3 | 66 | S4 LED S2 Lustr Direct Fan [10ch] 15˚- 30 ˚ | Bridge 2 DCS Font Light |
| 207 | 3 | 77 | S4 LED S2 Lustr Direct Fan [10ch] 15˚- 30 ˚ | Bridge 2 DSL Font Light |
| 208 | 3 | 88 | S4 LED S2 Lustr Direct Fan [10ch] 15˚- 30 ˚ | Bridge 2 DSL Font Light |
| 901 | 3 | 501 | Unique Haze | HL Balcony |
| 902 | 3 | 503 | Unique Haze | HR Balcony |
| 903 | 3 | 505 | MVS Haze [4ch] | USR on deck |
| 999 |  |  |  |  |
| (Part 2) | 1 | 401 | Houselights | HR String-Lights |
| (Part 3) | 1 | 402 | Houselights | HL String-Lights |
| (Part 4) | 1 | 403 | Houselights | Bridge 2 Birdies |
| (Part 5) | 1 | 404 | Houselights | Balconies & Doors |
| (Part 6) | 1 | 405 | Houselights | Stairs |
| (Part 7) | 1 | 406 | Houselights | Bridge 1 Birdies |
| (Part 8) | 1 | 407 | Houselights | Bubble Run |
| (Part 9) | 1 | 408 | Houselights |  |
| (Part 10) | 1 | 409 | Houselights |  |
| 1000 | 1 | 300-360 | Non-Dim @ 50 | Hot Power (16-Amps) |